# OEM / Dealer Battery Storage, Handling, Installation &

# 1. Purpose of This Training Guide

This document provides:

- Safe handling instructions

- Storage requirements

- Installation precautions

- Charging guidelines

- Maintenance recommendations

- Dealer/OEM best practices

- Safety and warranty protection guidelines

These procedures help improve:

- Battery safety

- Service life

- Installation quality

- Warranty performance

- Dealer support efficiency

# 2. Battery Charging Requirements

## Recommended Charging Voltage

- Charging voltage: 14.4V ±0.2V

- Use only compatible lithium battery chargers.

- Never reverse charging polarity.

## Charging Temperature

- Recommended charging temperature:

  - 0°C to 55°C (32°F to 131°F)

- Never charge below 0°C (32°F).

## Charging Safety

- Do not exceed the specified maximum charging current.

- Avoid prolonged overcharging.

- Disconnect charger after charging is complete.

- Use only approved chargers or charging systems.

# 3. Battery Operating Environment

## Discharge Temperature

Recommended operating range:

- -20°C to 60°C (-4°F to 140°F)

When operating above 45°C (113°F):

- Ensure proper ventilation

- Allow heat dissipation

- Avoid enclosed high-heat environments

## Humidity Requirements

- Recommended humidity: ≤85% RH

- Avoid condensation and standing water.

# 4. Battery Storage Requirements

## Recommended Storage Conditions

Store batteries in a:

- Dry area

- Clean area

- Well-ventilated environment

- Non-condensing environment

## Storage Temperature

- Recommended: 15°C–25°C (59°F–77°F)

- Acceptable range: -20°C to 60°C

## Keep Batteries Away From

- Fire

- Heaters

- Corrosive materials

- Flammable materials

- Water immersion

- Direct sunlight for prolonged periods

## Long-Term Storage Maintenance

Maintain battery voltage within:

- 13.28V–13.60V

Recommended maintenance intervals:

| Battery Type | Maintenance Interval |
| --- | --- |
| Standard battery | Every 2 months |
| Bluetooth / communication battery | At least once per month |

For long-term storage:

- Recharge batteries periodically

- Avoid prolonged low-voltage storage

- Follow FIFO (First-In First-Out) inventory practices

# 5. Inspection Before Installation

Upon receiving batteries:

## Inspect Packaging

Check for:

- Shipping damage

- Crushed cartons

- Water exposure

- Mishandling signs

## Inspect Battery Condition

Verify:

- No cracks

- No swelling

- No leakage

- No missing hardware

- No damaged terminals

## Electrical Inspection

Before installation:

- Verify correct polarity

- Confirm battery voltage is within operating range

- Ensure terminals are clean and dry

## Terminal Cleaning

If necessary:

- Use only a dry cloth

- Do not use solvents

- Do not use conductive cleaning materials

# 6. Installation Precautions

## Installation Environment

The installation area should be:

- Dry

- Clean

- Well ventilated

- Free from conductive debris

Avoid:

- Metal shavings

- Loose hardware

- Standing water

- Fuel vapors

- Open flames

## Terminal Connections

Before connecting:

- Confirm positive (+) and negative (-) polarity

- Verify cable routing

- Verify charger polarity

## Tightening Terminals

- Tighten terminals securely.

- Do not overtighten.

- Do not exceed specified torque values.

## Safety During Installation

- Avoid short circuits.

- Keep metal tools away from exposed terminals.

- Use insulated tools whenever possible.

- Remove jewelry before installation.

## Final Inspection

After installation:

- Verify all connections

- Confirm no loose hardware

- Ensure cables are secured

- Test equipment operation

- Observe for abnormal conditions

# 7. Battery Working Requirements

## Current Limits

Do not exceed:

- Maximum charging current

- Maximum discharge current

Excessive current may cause:

- Overheating

- Reduced service life

- BMS protection shutdown

- Internal circuit damage

- Safety hazards

## Low Voltage Operation

When battery voltage becomes low:

- Recharge promptly

- Avoid prolonged low-voltage storage

# 8. Daily Maintenance Recommendations

## Recommended Usage Practices

- Recharge batteries after use.

- Avoid deep discharge whenever possible.

- Shallow cycling may improve cycle life.

- Periodically inspect terminals and cables.

## Cleaning

- Use only a dry cloth.

- Do not use organic solvents.

- Do not spray water directly onto battery packs.

## Monitor for Abnormal Conditions

Immediately remove battery from service if any of the following occur:

- Excessive heat

- Swelling

- Smoke

- Odor

- Leakage

- Discoloration

- Cracking

- Unusual shutdown behavior

# 9. Strictly Prohibited Actions

The following actions are strictly prohibited:

- Reverse polarity connection

- Reverse charging

- Short-circuiting terminals

- Immersing battery in water

- Welding directly on terminals

- Piercing battery casing

- Crushing or dropping battery packs

- Disassembling battery packs

- Using battery above specified temperature limits

- Overloading beyond rated current

- Using damaged batteries

# 10. Transportation & Disposal

## Transportation Safety

Do not transport batteries together with:

- Loose metal objects

- Jewelry

- Conductive materials

Protect terminals during transportation.

## Disposal

Before disposal:

- Insulate exposed terminals

- Follow local recycling laws

- Send batteries to approved recycling facilities

## Transportation Safety

Do not transport batteries together with:

- Loose metal objects

- Jewelry

- Conductive materials

Protect terminals during transportation.

## Disposal

Before disposal:

- Insulate exposed terminals

- Follow local recycling laws

- Send batteries to approved recycling facilities

# 12. Additional Lynac / Kensun OEM & Dealer Recommendations

## Dealer Inventory Management

Recommended practices:

- Record serial numbers

- Record installation dates

- Monitor warehouse voltage periodically

- Rotate inventory using FIFO practices

## OEM Installation Best Practices

Recommended for marine, RV, and golf cart applications:

- Verify alternator compatibility with lithium batteries

- Verify charger compatibility

- Verify cable sizing

- Verify proper fuse protection

- Confirm correct polarity before startup

- Conduct final operational testing after installation

## Warranty Support Best Practices

To improve warranty processing efficiency:

Recommended documentation:

- Battery serial number

- Installation photos

- Wiring photos

- Voltage readings

- Charger model information

- Bluetooth screenshots (if available)

- Event/warning/fault codes (if available)

# 13. Dealer & Technician Quick Checklist

## Before Installation

☐ Inspect battery condition
☐ Verify voltage
☐ Confirm polarity
☐ Verify charger compatibility
☐ Inspect installation area

## During Installation

☐ Avoid short circuits
☐ Tighten terminals properly
☐ Keep area clean and dry
☐ Verify cable routing

## After Installation

☐ Test system operation
☐ Verify charging system
☐ Check for abnormal heat
☐ Record serial number
☐ Save installation photos

# 13. Reminder

For detailed chemical safety, fire response, hazardous material handling, transportation classification, and regulatory compliance information, refer to the official SDS/MSDS and applicable product certifications.

Always follow approved installation and operational procedures.